Serial Interrupt Manager Tool Set

---------------------------------

The Serial Interrupt Manager (SIM) is a code module that is installed by

a permanent init (PIF) file residing in the *:System:System.Setup

directory of an Apple IIGS.  SIM arbitrates serial interrupt vector

'patches' by providing a simple interface for applications and device

drivers to handle serial interrupts (from the IIGS's built-in SCC 8530)

in a manner completely consistent with Apple's guidelines on the subject.

SIM replaces the first quarter or so of the IIGS' main interrupt handler.

Most of the code is identical to that found in the ROM, with the

following notable exception: non-MIDI asynchronous serial interrupts

are handled by dispatching to routines that can be installed by using

the SIM tool.

SCC interrupt handlers installed through SIM are called, according

to Apple guidelines, immediately _after_ checking for AppleTalk and MIDI

interrupts.  This prevents AppleTalk or MIDI data loss, and provides the

lowest allowable overhead in processing asynchronous serial interrupts.

Data rates of up to 57600 baud are possible with no resulting data loss.

Since SIM handles serial interrupts through its own vector, and not

the $E10024 vector, the IIGS Serial Firmware can be used in combination

with SIM; SIM always has priority over the standard firmware handlers,

and if the firmware is used to initialize a port which is already being

handled by a SIM handler a crash may result.  Any suggestions for better

arbitration of the firmware are welcome.  (SIM could possibly check to see

if the firmware's SerFlag has been set, and automatically uninstall

the SIM handler and queue a warning dialog in the Scheduler.)

------------------------------------------------------------------------

SIM consists of a number of routines which are accessed via the

System 6.0 SendRequest IPC mechanism.  To reduce development times, SIM

comes with a library that can be used to access SIM in a language-

independent manner.

The following call descriptions show the various SIM calls in C format.

The calls expect their arguments in C order (i.e., pushing parameters

right to left).

The return value of the calls is either zero, indicating that no error

occurred, or a non-zero integer indicating the error number.  All of the

calls can return SIMNotFound in the event the SIM tool could not be located.

---------------------

int SIMVersion(void)

Returns the version number (in tool version format) of the SIM tool.

Errors:

SIMNotFound	- indicates that the SIM tool could not be located

($0006)		  Upon receiving this error, the application should

terminate attempting to install an interrupt

handler and should notify the user that the port

could not be opened.

---------------------

int InstallIntVect(word port, longword address)

InstallIntVect installs a serial handler routine into SIM's interrupt

dispatch code.  'address' is a pointer to an interrupt handler which

conforms to the serial interrupt handlers described in the Apple IIGS

Firmware Reference (which are called through the $E10024 vector).

The following steps should be taken to install a SIM handler:

Initialize SCC, but do not enable interrupts

call InstallIntVect

Enable SCC Interrupts

Errors:

SIMInvalidPort	An invalid port number was specified.  The only port numbers

($0005)		currently supported are SIMModemPort (2) and SIMPrinterPort (1)

SIMATalkActive	The specified port is already in use by AppleTalk, and thus

($0003)		cannot be used.

SIMAlreadyInst	Some other system component (a driver or application) already

($0001)		has an interrupt handler installed for that port.

In the last two cases, an application should notify the user that the port

is busy and cannot be used, and drivers should return an error code

appropriate to the condition.

---------------------

int RemoveIntVect(word port, longword address)

RemoveIntVect is used to uninstall a previously installed interrupt handler.

The port number and address of the handler must be specified.  This prevents

a program from trying to forcibly uninstall some other program's interrupt

handler, an approach which is not acceptable.

The procedure for uninstalling an interrupt handler should be as follows:

Turn of SCC interrupts and shutdown SCC chip if desired

Call RemoveIntVect

Errors:

SIMInvalidAddr	The address passed to RemoveIntVect does not match the

actual address of the handler on the specified port.

SIMNotInstalled	There is no handler currently installed for the specified

port.